# General Fire Inspection Hazards Checklist

- Building has address number (minimum of 4 inches) displayed so as to be plainly visible from the street.

- Buildings equipped with fire protection, detection systems, ANSUL cooking hood, or have an operating permit require the installation of a Knox Key Box.

- Exits/egress are not locked or obstructed from inside the building and are lighted. (keyed and surface bolts prohibited)

- Fire sprinkler/alarm and kitchen fire suppression systems are not impaired and have been serviced, tested and inspected by qualified tech. Records are available for inspection.

- Fire extinguishers are checked monthly and tagged annually.

- Employees are trained with fire extinguishers, fire protection equipment and emergency procedures and quarterly evacuation drills.

- Fire doors are not wedged or blocked open. Inspected annually for function and damage.

- Do not hang or cover ceilings or walls with combustible material/decorations.

- Open flame is not used in areas of assembly or egress.

- There are no open/exposed electrical wiring or hazards. Extension cords are not used on permanent appliances.

- Heating equipment operates properly. Only approved heating equipment in assembly space. Natural gas valves are accessible.

- Flammable liquids are not stored inside building.

- Trash and combustible waste must be stored in metal containers with a lid.

- Storage of combustibles - limited to 24 inches from ceiling without sprinkler or 18 inches from sprinkler equipped ceiling.

- Exit signage properly illuminated and emergency lighting functional. Requires 90-minute annual testing documentation.

- Elevator and boiler inspections up to date.

- CO alarms with 6-year battery are required one every 10,000sq ft per floor with source in commercial buildings.